V-Eye

Vision-Based Navigation System

for the Visually Impaired

Complete Deployment Guide

From Environment Setup to Full Pipeline Testing

Sensing and Testing Technology Course Project

School of Mechanical Engineering, Tongji University

June 2026

Table of Contents

1. Project Overview

1.1 Key Features

1.2 Original Paper

2. System Architecture

3. System Requirements

4. Environment Setup

4.1 Install System Packages

4.2 Install Pangolin

4.3 Install ORB-SLAM3

4.4 Clone and Build V-Eye

4.5 Verify Installation

5. Camera Setup and Calibration

5.1 Connect the Camera

5.2 Camera Intrinsic Calibration

5.3 Update Camera Configuration

6. SLAM Mapping

6.1 Prerequisites

6.2 Run SLAM Mapping

6.3 Output Files

6.4 model.bin Format

6.5 Tips for Good Mapping

7. Landmark Annotation

7.1 View the Point Cloud in CloudCompare

7.2 Create the Landmarks File

8. Full Pipeline Testing

8.1 Quick Start (Pre-built Assets)

8.2 Visualization Window Guide

8.3 Testing Individual Modules

9. Configuration Reference

9.1 configs/params.yaml

9.2 configs/orb_slam3.yaml

10. Directory Structure

11. Troubleshooting

11.1 Build Issues

11.2 Runtime Issues

11.3 Localization Quality Issues

11.4 General Tips

12. Complete Workflow Summary

1. Project Overview

V-Eye is a vision-based navigation system designed to assist the visually impaired. Originally published in IEEE Transactions on Multimedia (2021), this project reproduces the full system as part of the Sensing and Testing Technology course at Tongji University.

The system uses a HikRobot industrial camera to capture real-time video, applies ORB-SLAM3 for simultaneous localization and mapping (SLAM), performs global localization via model-based localization (MBL), fuses both position estimates through a variational Bayesian GPS (VB-GPS) integrator, and finally generates navigation guidance messages based on scene understanding (obstacle detection and distance estimation).

1.1 Key Features

- Real-time monocular SLAM using ORB-SLAM3

- Global localization via 2D-3D PnP matching (MBL)

- SLAM + MBL fusion for robust positioning (VB-GPS)

- Obstacle detection and monocular distance estimation

- Landmark-based navigation guidance

- Full pipeline visualization with real-time overlay

1.2 Original Paper

Duh, P.-J., Sung, Y.-C., Chiang, L.-Y. F., Chang, Y.-J., & Chen, K.-W. (2021). "V-Eye: A Vision-Based Navigation System for the Visually Impaired." IEEE Transactions on Multimedia, 23, 1567-1580.

2. System Architecture

The following diagram illustrates the data flow between modules:

HikRobot Camera  -->  ORB-SLAM3  -->  VB-GPS Integrator  -->  Navigator  -->  Output

(io/)              (relative      (SLAM + MBL            (landmark      (direction +

pose +        fusion)                query +        distance +

keyframes)                           msg generation) action)

Module Descriptions

| Module | Description |
| --- | --- |
| io | Camera I/O layer. HikRobot driver for frame capture, USB recovery, and Bayer-to-RGB conversion. |
| tools | Utility library. Math tools (Euler/quaternion/transform), image drawing, UDP plotter, spdlog logging, SIGINT handler, thread-safe queue, MP4 recorder, YAML I/O. |
| nav | Navigation core. ORB-SLAM3 wrapper, MBL global localization (2D-3D PnP + RANSAC), VB-GPS integrator (SLAM + MBL fusion), scene understanding (edge detection + monocular ranging), landmark map, navigator (guidance messages). |
| src | Executable entry points: run_slam, test_mbl, test_vb_gps, test_scene, test_full, record_video. |
| calibrate | Calibration programs: image capture, camera intrinsic calibration, hand-eye calibration. |
| configs | Configuration files: params.yaml (calibration + camera), orb_slam3.yaml (ORB-SLAM3 settings). |
| assets | Resource files: video samples, 3D models (map.ply + model.bin), ORB vocabulary, landmark annotations. |

3. System Requirements

3.1 Operating System

- Ubuntu 22.04 or newer (recommended)

- Other Linux distributions may work with adjustments to package names

3.2 Hardware

- CPU: x86_64, 4+ cores recommended

- RAM: 8 GB minimum (ORB vocabulary file requires ~1.5 GB to load)

- Camera: HikRobot USB industrial camera (VID/PID: 2bdf:0001)

- GPU: Not required (CPU-only pipeline)

3.3 Software Dependencies

| Dependency | Version | Purpose |
| --- | --- | --- |
| CMake | ≥ 3.16 | Build system |
| GCC / G++ | ≥ 9 | C++17 compiler |
| OpenCV | ≥ 4.x | Image processing, feature extraction |
| Eigen3 | ≥ 3.3 | Linear algebra |
| Pangolin | latest | 3D visualization (ORB-SLAM3 viewer) |
| ORB-SLAM3 | latest | Visual SLAM framework |
| spdlog + fmt | latest | Logging + string formatting |
| yaml-cpp | latest | YAML config file parsing |
| nlohmann_json | latest | JSON landmark file parsing |
| FFmpeg (libav*) | latest | Video decoding |
| Boost (serialization) | latest | ORB-SLAM3 dependency |
| OpenSSL | latest | ORB-SLAM3 dependency |
| libusb | latest | USB camera communication |
| OpenVINO | 2024.6.0 | AI inference (optional) |
| Git LFS | latest | Large file storage (video, model files) |

4. Environment Setup

This chapter guides you through installing all dependencies and building the project from source. Follow each step in order.

4.1 Install System Packages

Open a terminal and run the following commands:

sudo apt update && sudo apt upgrade -y

# Build tools

sudo apt install -y build-essential cmake pkg-config git

# OpenCV

sudo apt install -y libopencv-dev libopencv-contrib-dev

# Eigen3

sudo apt install -y libeigen3-dev

# Logging and serialization

sudo apt install -y libspdlog-dev libfmt-dev libyaml-cpp-dev nlohmann-json3-dev

# FFmpeg

sudo apt install -y libavcodec-dev libswscale-dev libavutil-dev

# USB support

sudo apt install -y libusb-1.0-0-dev

# Pangolin dependencies (OpenGL)

sudo apt install -y libglew-dev libepoxy-dev libgl1-mesa-dev

# ORB-SLAM3 dependencies

sudo apt install -y libboost-serialization-dev libssl-dev

# Optional tools

sudo apt install -y cloudcompare

# Git LFS

sudo apt install -y git-lfs

git lfs install

4.2 Install Pangolin

Pangolin is a lightweight OpenGL-based 3D visualization library required by ORB-SLAM3.

git clone https://github.com/stevenlovegrove/Pangolin.git --depth 1

cd Pangolin

cmake -B build -DCMAKE_BUILD_TYPE=Release -DBUILD_PANGOLIN_PYTHON=OFF

cmake --build build -j$(nproc)

sudo cmake --build build --target install

sudo ldconfig

cd ..

4.3 Install ORB-SLAM3

ORB-SLAM3 must be patched for C++17 compatibility before building.

Step 1: Clone the repository

git clone https://github.com/UZ-SLAMLab/ORB_SLAM3.git --recursive --depth 1 /tmp/ORB_SLAM3

Step 2: Apply required patches

Edit /tmp/ORB_SLAM3/CMakeLists.txt and change the C++ standard check from C++11 to C++17:

# Find the line checking for C++11 and replace with:

set(CMAKE_CXX_STANDARD 17)

Edit /tmp/ORB_SLAM3/include/LoopClosing.h, line 226, and change the type of mnFullBAIdx:

# Find:

bool mnFullBAIdx

# Replace with:

int mnFullBAIdx

Step 3: Build

cd /tmp/ORB_SLAM3

chmod +x build.sh

./build.sh

4.4 Clone and Build V-Eye

Step 1: Clone the project and pull LFS files

cd ~

git clone <your-repo-url> v_eye

cd v_eye

# Pull large files (video, ORB vocabulary) managed by Git LFS

git lfs pull

Step 2: Build the project

cmake -B build -S . -DCMAKE_BUILD_TYPE=Release

cmake --build build -j$(nproc)

After a successful build, the following executables are available in build/:

| Executable | Description |
| --- | --- |
| build/run_slam | ORB-SLAM3 mapping from video or image sequence |
| build/test_mbl | Test MBL global localization (2D-3D PnP) |
| build/test_vb_gps | Test VB-GPS fusion (SLAM + MBL) |
| build/test_scene | Test scene understanding (obstacle + distance) |
| build/test_full | Full pipeline with visualization (requires GUI) |
| build/record_video | Record video from HikRobot camera |
| build/capture_withoutq | Capture calibration images |
| build/calibrate_camera | Camera intrinsic calibration |
| build/calibrate_device2world_handeye | Hand-eye + world coordinate calibration |

4.5 Verify Installation

Run the following tests to verify each component works correctly:

# Test MBL global localization (no GUI required)

./build/test_mbl

# Test scene understanding (opens GUI window)

./build/test_scene

# Test full pipeline (opens GUI window, needs display)

./build/test_full

5. Camera Setup and Calibration

If you are using a pre-recorded video, you can skip this chapter. This chapter is only needed when working with a live HikRobot camera.

5.1 Connect the Camera

- Connect the HikRobot USB camera to your computer

- Verify it is detected: lsusb | grep 2bdf

- The expected VID/PID is 2bdf:0001 (configurable in configs/params.yaml)

5.2 Camera Intrinsic Calibration

Step 1: Capture calibration images

Use a checkerboard pattern (10 columns x 7 rows, 40 mm spacing). Run the capture program and press S to save each image. Capture at least 15-20 images from different angles.

./build/capture_withoutq

# Press S to save an image

# Press ESC to exit when done

Step 2: Run calibration

./build/calibrate_camera

The calibration program outputs camera intrinsics (fx, fy, cx, cy) and distortion coefficients (k1, k2, p1, p2, k3). These are automatically written to configs/params.yaml and configs/orb_slam3.yaml.

Step 3: Verify calibration quality

Check the reprojection error in the output. Values below 0.5 pixels indicate good calibration. The provided configuration has a reprojection error of 0.1328 px.

5.3 Update Camera Configuration

If calibration values differ, update configs/orb_slam3.yaml with the new parameters:

Camera1.fx: <your_fx>

Camera1.fy: <your_fy>

Camera1.cx: <your_cx>

Camera1.cy: <your_cy>

Camera1.k1: <your_k1>

Camera1.k2: <your_k2>

Camera1.p1: <your_p1>

Camera1.p2: <your_p2>

Camera1.k3: <your_k3>

Camera.width: 1440

Camera.height: 1080

Camera.fps: 30

6. SLAM Mapping

This chapter explains how to build a 3D point cloud map from a video file or image sequence. The output serves as the reference model for MBL global localization.

6.1 Prerequisites

- A video file (.mp4) or a directory of sequential images

- Sufficient disk space for the output model files (typically 50-500 MB)

- The ORB vocabulary file: assets/ORBvoc.txt (139 MB, managed by Git LFS)

6.2 Run SLAM Mapping

Video mode (recommended):

./build/run_slam assets/basement.mp4 assets/colmap_basement

Image sequence mode:

# Point to a directory (no file extension)

./build/run_slam /path/to/image_directory assets/colmap_basement

The mapping process will:

- Extract ORB features from each frame

- Track features and estimate camera poses

- Triangulate 3D points to build a sparse point cloud

- Perform local bundle adjustment for accuracy

- Detect loops and perform global bundle adjustment

6.3 Output Files

Two files are generated in the output directory:

| File | Format | Purpose |
| --- | --- | --- |
| map.ply | PLY text | Point cloud for visualization in CloudCompare or MeshLab. Contains 3D coordinates and colors. |
| model.bin | Binary | MBL model file for global localization. Contains 3D points with ORB descriptors. |

6.4 model.bin Format

The binary model file has the following structure:

[uint32_t]  Number of 3D points (N)

For each point (N times):

[float x3]   X, Y, Z coordinates (world frame)

[uint8 x32]  ORB descriptor (256-bit)

6.5 Tips for Good Mapping

- Use a well-lit environment with rich visual texture

- Move the camera slowly and steadily

- Avoid sudden rotations or fast motion

- Include loop closures (return to previously seen areas)

- Cover the entire area you want to navigate later

- Trim video to remove static or irrelevant segments before mapping

7. Landmark Annotation

Landmarks define key locations in the mapped environment where navigation guidance should be provided. Each landmark includes a 3D position, orientation, and navigation instructions.

7.1 View the Point Cloud in CloudCompare

CloudCompare assets/colmap_basement/map.ply

In CloudCompare, click on key locations in the point cloud to get their X, Y, Z coordinates. Note these down for landmark creation.

7.2 Create the Landmarks File

Edit assets/landmarks_basement.json (or create a new JSON file). Each landmark entry contains:

| Field | Type | Description |
| --- | --- | --- |
| x, y, z | float | 3D coordinates in the world frame (from CloudCompare) |
| yaw | float | Yaw angle in degrees for the facing direction |
| on_left | string | Description of what is on the left side |
| on_right | string | Description of what is on the right side |
| caution | string | Warning or caution message (empty string if none) |
| action | string | Navigation action: go straight, turn left/right/back, etc. |

Example landmark:

{

"x": -0.03, "y": 0.05, "z": 0.04, "yaw": 0,

"on_left": "wall",

"on_right": "corridor",

"caution": "",

"action": "go straight"

}

Tips for landmark placement:

- Place landmarks at decision points: intersections, turns, doors

- Ensure landmarks are within the mapped area (visible in the point cloud)

- Use clear, concise descriptions for navigation messages

- Set appropriate yaw angles for the direction the user faces at each landmark

8. Full Pipeline Testing

Once the 3D map, model file, and landmarks are ready, you can run the complete navigation pipeline.

8.1 Quick Start (Pre-built Assets)

Using the provided basement scene assets:

# Run the full pipeline with visualization

./build/test_full assets/colmap_basement/model.bin \

assets/basement.mp4 \

assets/landmarks_basement.json

8.2 Visualization Window Guide

When test_full runs, it opens a GUI window showing real-time information:

| Color | Element | Meaning |
| --- | --- | --- |
| White text | SLAM status | SLAM pose, fused pose, keyframe count, tracking state (OK/LOST) |
| Yellow boxes | Obstacles | Detected obstacles with distance labels |
| Green text | Navigation guidance | Direction and action messages for the user |
| Warnings | Warnings | Obstacle warnings and collision alerts |

Press ESC to exit the visualization.

8.3 Testing Individual Modules

Each module can be tested independently for debugging purposes.

8.3.1 MBL Global Localization (test_mbl)

Tests 2D-3D feature matching for global camera localization.

./build/test_mbl assets/colmap_basement/model.bin assets/basement.mp4

Output: Every 30 frames, prints inlier count, feature distribution area, and global pose. A reliable localization has:

- Inlier count ≥ 40

- Distribution area ≥ 1/8 of the frame area

8.3.2 VB-GPS Fusion (test_vb_gps)

Runs SLAM and MBL simultaneously, then fuses their pose estimates.

./build/test_vb_gps assets/colmap_basement/model.bin assets/basement.mp4

Output: Every 30 frames, prints fusion status (Y = fused / N = not fused) and the fused pose.

8.3.3 Scene Understanding (test_scene)

Tests obstacle detection and monocular distance estimation.

./build/test_scene

Output: GUI window showing yellow bounding boxes around detected obstacles with distance labels. Press ESC to exit.

9. Configuration Reference

9.1 configs/params.yaml

Main configuration file for camera parameters, calibration, and recording.

| Parameter | Example Value | Description |
| --- | --- | --- |
| pattern_cols | 10 | Checkerboard pattern columns for calibration |
| pattern_rows | 7 | Checkerboard pattern rows |
| center_distance_mm | 40 | Checkerboard square size in mm |
| camera_name | hikrobot | Camera identifier |
| exposure_ms | 11 | Camera exposure time in ms |
| gain | 10.0 | Camera gain |
| vid_pid | 2bdf:0001 | USB Vendor ID : Product ID |
| Camera1.fx / fy | 2601.58 | Focal length in pixels (x / y) |
| Camera1.cx / cy | 778.77 | Principal point in pixels |
| Camera1.k1/k2/p1/p2/k3 | see file | Distortion coefficients (radial + tangential) |
| Camera.width / height | 1440 / 1080 | Image resolution in pixels |
| Camera.fps | 30 | Camera frame rate |
| output_path | assets/output.mp4 | Output path for recorded video |

9.2 configs/orb_slam3.yaml

ORB-SLAM3-specific configuration. Key parameters:

| Parameter | Example Value | Description |
| --- | --- | --- |
| Camera.type | PinHole | Camera model type |
| ORBextractor.nFeatures | 2000 | Number of ORB features to extract per frame |
| ORBextractor.scaleFactor | 1.2 | Scale factor between pyramid levels |
| ORBextractor.nLevels | 8 | Number of scale pyramid levels |
| ORBextractor.iniThFAST | 20 | Initial FAST corner detection threshold |
| ORBextractor.minThFAST | 7 | Minimum FAST corner detection threshold |
| Viewer.KeyFrameSize | 0.05 | Keyframe display size in viewer |
| Viewer.ViewpointX/Y/Z | 0/-0.7/-1.8 | Initial 3D viewer viewpoint |

10. Directory Structure

For reference, the complete project layout:

| Path | Contents |
| --- | --- |
| CMakeLists.txt | Top-level CMake build file |
| README.md | Project overview and quick start guide |
| AGENTS.md | Developer documentation with architecture details |
| configs/ | Config files (params.yaml, orb_slam3.yaml) |
| io/ | Camera I/O: HikRobot driver, frame capture, Bayer-to-RGB |
| tools/ | Utilities: math, image drawing, logging, signal handling, video recording, YAML I/O |
| nav/ | Navigation: ORB-SLAM3 wrapper, MBL, VB-GPS, scene understanding, landmark map, navigator |
| src/ | Executable program entry points |
| calibrate/ | Calibration programs |
| assets/ | Assets: videos, models (.ply/.bin), vocabulary, landmarks (.json) |
| build/ | CMake build output |
| logs/ | Runtime logs |

11. Troubleshooting

11.1 Build Issues

CMake cannot find OpenCV

sudo apt install -y libopencv-dev libopencv-contrib-dev

CMake cannot find Pangolin

# Ensure Pangolin is installed and ldconfig is run:

sudo ldconfig

pkg-config --modversion pangolin  # should print version

ORB-SLAM3 compilation errors (C++ standard mismatch)

# Verify you changed CMakeLists.txt to C++17:

grep CMAKE_CXX_STANDARD /tmp/ORB_SLAM3/CMakeLists.txt

# Verify LoopClosing.h fix:

grep mnFullBAIdx /tmp/ORB_SLAM3/include/LoopClosing.h

Missing nlohmann_json / yaml-cpp

sudo apt install -y nlohmann-json3-dev libyaml-cpp-dev

11.2 Runtime Issues

Segmentation fault on startup

- Verify ORBvoc.txt is present: ls -lh assets/ORBvoc.txt

- If missing, run: git lfs pull

- The file should be approximately 139 MB

GUI window not opening (headless server)

- For headless servers without a display, use test_mbl or test_vb_gps

- test_full and test_scene require a running X11 display

- You can use X11 forwarding: ssh -X user@server

Video file cannot be opened

- Verify the video path is correct and the file exists

- Ensure FFmpeg libraries are installed: sudo apt install -y libavcodec-dev libswscale-dev libavutil-dev

- Check that the video codec is supported (H.264 recommended)

Camera not detected (record_video)

- Check USB connection: lsusb | grep 2bdf

- Verify VID/PID in configs/params.yaml matches your camera

- Ensure libusb is installed: sudo apt install -y libusb-1.0-0-dev

- Check USB permissions; you may need udev rules for the HikRobot camera

11.3 Localization Quality Issues

Poor MBL localization (low inlier count)

- Ensure the 3D model covers the area being navigated

- Increase ORBextractor.nFeatures in orb_slam3.yaml (e.g., from 2000 to 3000)

- Verify the camera intrinsics in orb_slam3.yaml are correct

- Make sure the environment has sufficient visual texture

SLAM tracking lost frequently

- Reduce camera motion speed

- Ensure adequate lighting

- Lower ORBextractor.iniThFAST from 20 to 10 for more features

11.4 General Tips

- Always build in Release mode for performance

- Check build/logs/ for detailed error logs

- Use git lfs pull after cloning to download all large files

- Run cmake --build build -j$(nproc) to use all CPU cores for faster compilation

12. Complete Workflow Summary

Below is the end-to-end workflow to deploy V-Eye on a new environment:

Phase 1: Environment Setup

- Install system dependencies (apt packages)

- Build and install Pangolin from source

- Clone, patch, and build ORB-SLAM3

- Clone V-Eye, pull LFS files, and build with CMake

- Verify with test_mbl and test_scene

Phase 2: Camera Calibration (if using live camera)

- Connect HikRobot camera and verify USB detection

- Capture calibration images with capture_withoutq

- Run calibrate_camera and update config files

Phase 3: Mapping

- Record or obtain a video of the target environment

- Run run_slam to build the 3D map and MBL model

- Verify map quality by viewing map.ply in CloudCompare

Phase 4: Landmark Annotation

- Open map.ply in CloudCompare and note key coordinates

- Create a landmarks JSON file with positions and navigation messages

Phase 5: Deployment and Testing

- Run test_full with the model, video, and landmarks

- Verify SLAM tracking, MBL localization, and navigation messages

- Test with test_mbl and test_vb_gps for module-level debugging if needed

V-Eye Project — Sensing and Testing Technology — School of Mechanical Engineering, Tongji University — June 2026